Turn on/off LED with Blynk

- Requirements

- Use Wemos D1 wifi board

- Setup Blynk app on your smart phone

- Use Blynk to turn ON/OFF the LED

- Devices:

| Devices | Description |
| --- | --- |
| WeMos D1 board |  |
| LED |  |
| Resistor |  |

- Blynk installation

Download Blynk from Apple AppStore or Google Play Store

- Sign up for a Blynk account

Create new project (Note: Token will be sent toyour email email)

Select hardware: WeMos D1

Add a button on Blynk

- Install Blynk library

(How to install Blynk library: http://help.blynk.cc/en/articles/512105-how-to-install-blynk-library-for-arduino-ide)

- Board Setup

- From the Arduino IDE, select File>Preferences in menu bar

- Enter http://arduino.esp8266.com/stable/package_esp8266com_index.json in the Additional Board Manager URLs box.

- SelectTools> Board menu >  Boards Manager  and  search for esp8266 platform.

- Select the version from Drop-down list.

- Click install.

- The Schematic

- We use the D2 pin for LED controlling.

- Code the sketch

#define BLYNK_PRINT Serial

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

// You should get Auth Token in the Blynk App.

char auth[] = "YOUR AUTH TOKEN KEY";

// Your WiFi credentials.

// Set password to Wifi networks.

char ssid[] = "Greenwich-Student";

char pass[] = "12345678";

void setup()

{

// Debug console

Serial.begin(9600);

Blynk.begin(auth, ssid, pass);

//Setup LED

pinMode(D2, OUTPUT);

}

//Setup for Blynk App to control Led

BLYNK_WRITE(V1)

{

int pinValue = param.asInt();

if(pinValue == 1)

{

digitalWrite(D2, HIGH);

}

else

{

digitalWrite(D2, LOW);

}

}

void loop()

{

Blynk.run();

}